Series Circuits

For each resistor, determine:

the current through the resistor

the voltage drop across the resistor

the resistance of each resistor

For the entire circuit, determine:

the total resistance

the total voltage of the circuit

the total current supplied by the battery

- A 12.0 V DC battery is connected in series with two resistors, as shown in the diagram:

- Resistor R1=5.0 Ω

- Resistor R2=7.0 Ω

Assume all components are ideal and connecting wires have negligible resistance.

- A series circuit consists of three resistors connected to a DC power source, as shown in the diagram:

- R1=15 Ω

- R2=25 Ω

- R3=35 Ω

The total current supplied by the source is Itotal=30 A Assume all components are ideal and connecting wires have negligible resistance.

- A 1.5 V DC battery is connected in series with three resistors, as shown in the circuit diagram. The following information is known:

- R1=65 Ω

- The voltage drop across R2is V2=0.40 V

- The voltage drop across R3is V3=0.30 V

Assume all components are ideal and the resistance of connecting wires is negligible.

- A series circuit consists of two resistors, R1and R2, connected to a DC power source, as shown in the diagram. The following information is known:

- The total current in the circuit is Itotal=45 mA

- The voltage drop across R1is V1=1.0 V

- The voltage drop across R2is V2=8.0 V

Assume all components are ideal and the resistance of the connecting wires is negligible.

Parallel Circuits

- A 4.5 V DC battery is connected to two resistors in parallel, as shown in the diagram:

- R1=15 Ω

- R2=25 Ω

Assume all components are ideal and the resistance of the connecting wires is negligible.

- A DC power source is connected to two resistors in parallel, as shown in the circuit diagram. The following information is known:

- The total current supplied by the source is Itotal=4.5 A

- The voltage across R1is V1=120 V

- The resistance of the second resistor is R2=50 Ω

Assume all components are ideal and the resistance of the connecting wires is negligible.

- A 6.0 V DC battery is connected to two resistors in parallel, as shown in the circuit diagram. The following information is known:

- The current through resistor R1is I1=64 mA

- The current through resistor R2is I2=32 mA

Assume all components are ideal and the resistance of the connecting wires is negligible.

- A 9.0 V DC battery is connected to three resistors in parallel, as shown in the circuit diagram:

- R1=2.0 Ω

- R2=4.0 Ω

- R3=6.0 Ω

Assume all components are ideal and that the resistance of the connecting wires is negligible.