Circuits & Waves Review

A 9.0 V battery is connected in series with two resistors, R1=3.0 Ωand R2=6.0 Ω.

- Determine:

- (a) total resistance

- (b) total current

- (c) voltage drop across each resistor

A 12.0 V battery is connected to two resistors in parallel, R1=4.0 Ωand R2=12.0 Ω.

- Determine:

- (a) equivalent resistance

- (b) total current

- (c) current through each resistor

Two resistors are connected in series. The total current in the circuit is 2.0 A. If the voltage drops across the resistors are 4.0 V and 8.0 V, determine:

- (a) the resistance of each resistor

- (b) the total voltage of the source

A 6.0 V battery is connected to two resistors in parallel. The currents through the resistors are 0.50 A and 1.0 A.

- Determine:

- (a) the resistance of each resistor

- (b) the total resistance of the circuit

A 12.0 V battery is connected to:

- a 5.0 Ω resistor in series with

- a parallel combination of 10.0 Ω and 20.0 Ω

Determine:

(a) equivalent resistance of the parallel section

(b) total resistance

(c) total current

(d) current through each resistor

A circuit consists of two branches in parallel:

- Branch A: a single 8.0 Ω resistor

- Branch B: two resistors in series, 3.0 Ω and 5.0 Ω

The circuit is connected to a 16.0 V battery. Determine:

(a) total resistance

(b) current in each branch

(c) current through each resistor

A 9.0 V battery is connected to a circuit where:

- one branch has a 6.0 Ω resistor

- the other branch has two 3.0 Ω resistors in series

Determine:

(a) total resistance

(b) total current

(c) current through each resistor

A combination circuit has a total resistance of 4.0 Ω and is connected to a 12.0 V battery.

- Determine:

(a) the total current supplied by the battery

(b) the voltage across the parallel section of the circuit

A real battery has an emf of 12.0 V and an internal resistance of 1.5 Ω. It is connected to a single external resistor of 6.0 Ω.

- Determine:

(a) total resistance

(b) current

(c) terminal voltage

A real battery with emf 18.0 V and internal resistance 1.0 Ω is connected to two external resistors in series:

- R1=4.0 Ω, R2=5.0 Ω.

Determine:

(a) total current

(b) terminal voltage

(c) voltage drop across each external resistor

A battery with emf 24.0 V supplies current to two resistors in parallel: 6.0 Ω and 12.0 Ω. The terminal voltage is measured to be 18.0 V.

Determine:

(a) total current supplied

(b) the internal resistance of the battery

A battery has an emf of 15.0 V and internal resistance 2.0 Ω. It is connected to two resistors in parallel: 4.0 Ω and 12.0 Ω.

Determine:

(a) equivalent resistance of the external circuit

(b) total current

(c) power dissipated in the internal resistance

Classify each as:

- pulse or continuous

- transverse or longitudinal

- mechanical or electromagnetic

(a) A single ripple moving across a pond

(b) A sound wave from a speaker

(c) Light traveling through glass

A wave has a frequency of 5.0 Hz. Determine:

(a) the period

(b) how many cycles occur in 12 seconds

A wave travels at 3.0 m/s and has a wavelength of 0.60 m. Determine:

(a) frequency

(b) period

Ocean waves pass a buoy at a rate of 10 crests in 20 s. The distance between crests is 4.0 m. Determine:

(a) frequency

(b) wave speed

A mass oscillates with an amplitude of 4.0 cm and a period of 2.0 s. Assume it passes through equilibrium moving upward at t=0.

(a) Write the displacement function

(b) Determine the first time the mass reaches maximum height

A spring oscillates with a frequency of 3.0 Hz and amplitude 2.0 cm. Determine:

(a) the period

(b) the displacement function

(c) how many times it reaches maximum displacement in 2.0 s

A wave has an amplitude of 5.0 cm and wavelength 1.0 m. Assume the wave passes through equilibrium at x=0and is increasing.

(a) Write the spatial wave equation
(b) Find the position of the first crest

A transverse wave travels along a rope with wavelength 0.80 m. Determine:

(a) the distance between consecutive zero crossings

(b) the distance from equilibrium to the first crest

A siren emits a frequency of 600 Hz. It moves toward a stationary observer at 20 m/s. Assume the speed of sound is 340 m/s.

(a) Determine the observed frequency

(b) Explain why the frequency increases

Light travels from air into water at an angle of incidence of 40°.

- The refractive index of water is 1.33.

(a) Determine the angle of refraction

(b) Explain why the ray bends toward the normal

A wave has a frequency of 2.0 Hz and wavelength 1.5 m.

(a) Determine the wave speed

(b) Sketch one full wavelength and label amplitude and wavelength