2- Combination Circuits

- A 6.0 V DC battery is connected to a circuit consisting of three resistors, as shown in the diagram.

- R1=5.0 Ω is connected in series with a parallel combination of:

  - R2=5.0 Ω

  - R3=5.0 Ω

(a) Determine the equivalent resistance of the parallel section containing R2and R3.

(b) Determine the total resistance of the circuit.

(c) Determine the total current supplied by the battery.

(d) Determine the current through R1.

(e) Determine the current through R2and R3.

(f) Determine the voltage drop across each resistor.

- A 12.0 V DC battery is connected to a circuit consisting of three resistors, as shown in the diagram.

- R3=360 Ωis connected in series with a parallel combination of:

  - R1=240 Ω

  - R2, whose resistance is unknown

    - The voltage drop across R2is V2=4.4 V

(a) Determine the voltage across the parallel section of the circuit.

(b) Determine the current through R1.

(c) Determine the current through R2.

(d) Determine the resistance of R2.

(e) Determine the equivalent resistance of the parallel section.

(f) Determine the total resistance of the circuit.

(g) Determine the total current supplied by the battery.

- A 180 V DC power source is connected to a circuit consisting of four resistors, arranged as shown in the diagram.

- R1=64 Ωand R4=72 Ωare connected in series with the rest of the circuit.

- Between the top and bottom nodes, there are two resistors in parallel:

  - R2, carrying a current of I2=0.40 A

  - R3, carrying a current of I3=0.60 A

(a) Determine the total current supplied by the power source.

(b) Determine the voltage across the parallel section of the circuit.

(c) Determine the voltage across R2.

(d) Determine the resistance of R2.

(e) Determine the resistance of R3.

(f) Verify that your results are consistent with Ohm’s Law and the rules for series–parallel circuits.

- A DC power source is connected to a circuit consisting of three resistors, arranged as shown in the diagram.

- The total current supplied by the source is Itotal=58 mA

- One branch contains a single resistor:

  - R1=3800 Ω

  - The voltage across this resistor is V1=120 V

- The other branch contains two resistors in series:

  - R2=3200 Ω

  - R3, whose resistance is unknown

(a) Determine the voltage of the power source.

(b) Determine the current through R1.

(c) Determine the current through the branch containing R2and R3.

(d) Determine the voltage across the series combination of R2and R3.

(e) Determine the resistance of R3.

- A circuit consists of a DC power source connected to four resistors, arranged as shown in the diagram.

- R1=5.4 Ωis connected in series with a parallel network of three resistors:

  - R2=45 Ω

  - R3=39 Ω

  - R4=60 Ω, The voltage across R4is V4=6.2 V

- The voltage of the power source, VT, is unknown.

(a) Determine the voltage across the parallel section of the circuit.

(b) Determine the current through R4.

(c) Determine the current through R2and R3.

(d) Determine the total current entering the parallel section.

(e) Determine the voltage drop across R1.

(f) Determine the voltage of the power source, VT.

Important Instructions

- Do not substitute numerical values for R.

- All answers must be expressed symbolically in terms of R.

- Final expressions should be fully simplified.

- A 120 V DC power source is connected to a circuit consisting of four identical resistors, each with resistance R, arranged as shown in the diagram.

- Two resistors are connected in series along the top branch.

- One resistor is connected alone in the middle branch.

- One resistor is connected in series on the right side of the circuit.

- The current through the middle resistor is labeled I1.

(a) Determine the equivalent resistance of the top branch in terms of R.

(b) Determine the equivalent resistance of the parallel section in terms of R.

(c) Determine the total resistance of the circuit in terms of R.

(d) Determine the total current supplied by the battery in terms of R.

(e) Determine the current through the middle resistor, I1, in terms of R.

- A 6.0 V DC power source is connected to a circuit consisting of four resistors, arranged as shown in the diagram. The circuit is made of two parallel sections connected in series:

Lower parallel section

- R1=15 kΩ, The current through R1is I1=1.5 mA

- R2=35 kΩ

Upper parallel section

- R3=3.0 kΩ

- R4, whose resistance is unknown

(a) Determine the voltage across the lower parallel section.

(b) Determine the current through R2.

(c) Determine the total current flowing through the circuit.

(d) Determine the voltage across the upper parallel section.

(e) Determine the current through R3.

(f) Determine the resistance of R4.

- An 18.0 V DC battery is connected to a circuit consisting of four resistors, arranged as shown in the diagram. The circuit has two branches in parallel:

Upper branch

- A resistor R2=84 Ωis connected in series with a parallel combination of:

  - R3=6.5 Ω

  - R4=4.0 Ω

Lower branch

- A single resistor R1=10.3 Ω

The total current supplied by the battery, IT, is unknown.

(a) Determine the equivalent resistance of the parallel pair R3and R4.

(b) Determine the total resistance of the upper branch.

(c) Determine the equivalent resistance of the entire circuit.

(d) Determine the total current supplied by the battery.

(e) Determine the current in each branch.