More Circuits

- A DC power source is connected to a circuit consisting of four resistors, arranged as shown in the diagram.

- Resistors R1and R2are connected in series, forming one branch of the circuit.

- Resistors R3and R4are connected in series, forming a second branch of the circuit.

- The two series branches are connected in parallel with each other.

The following information is known:

- The current through resistor R1is IR1=5.0 A

- The current in the left vertical wire below the junction is I=8.0 A

- The voltage drop across resistor R2is VR2=70 V

- R3=3.0 Ω

(a) Determine the current in each branch of the circuit.

(b) Determine the current through R2.

(c) Determine the resistance of R2.

(d) Determine the current through R3and R4.

(e) Determine the voltage across each series branch.

(f) Determine the resistance of R1.

(g) Determine the voltage of the power source.

- A 6.0 V DC battery is connected to a circuit consisting of four resistors, arranged as shown in the diagram.

- R1=100 Ωis connected in series with the rest of the circuit.

- After R1, the circuit splits into three parallel branches:

  - R2=200 Ω

  - R3=300 Ω

  - R4=400 Ω

All three of these resistors are connected between the same two junctions, and therefore form a parallel combination.

(a) Determine the equivalent resistance of the parallel combination of R2, R3, and R4.

(b) Determine the total resistance of the circuit.

(c) Determine the total current supplied by the battery.

(d) Determine the current through each resistor: R1, R2, R3, and R4.

(e) Determine the voltage drop across each resistor.

(f) Verify that your results are consistent with Ohm’s Law and the rules for series and parallel circuits.

- An 8.0 V DC battery is connected to a circuit consisting of four resistors, arranged as shown in the diagram.

- R1=40 Ωis connected in series with the rest of the circuit.

- R2=60 Ωand R3=120 Ωare connected in parallel with each other.

- Resistor R4=40 Ωis bypassed by a conducting wire, creating a short circuit across it.

(a) Explain why R4has no effect on the behaviour of the circuit.

(b) Determine the equivalent resistance of the parallel combination of R2and R3.

(c) Determine the total resistance of the circuit.

(d) Determine the total current supplied by the battery.

(e) Determine the current through R2and R3.

(f) Determine the voltage drop across R1, R2, and R3.

- A 12.0 V DC power source is connected to a circuit consisting of five resistors, arranged as shown in the diagram.

- Lower branch:

  - R1is connected in series with R2.

  - The voltage drop across R1is 5.0 V.

  - The current through R2is 6.0 A.

- Upper branch:

  - A total current of 4.0 A enters a parallel combination of:

    - R3, which carries 3.0 A

    - R4, which has a voltage drop of 8.0 V

  - The parallel combination then reconnects and passes through R5in series.

- The upper and lower branches are connected in parallel between the same two junctions.

(a) Determine the current through R4.

(b) Determine the current through R5.

(c) Determine the resistance of R4.

(d) Determine the resistance of R3.

(e) Determine the resistance of R1.

(f) Determine the resistance of R2.

(g) Determine the voltage across the upper branch.

- A 12.0 V DC battery is connected to a circuit consisting of six resistors, arranged as shown in the diagram. All resistors have the same resistance:

R1=R2=R3=R4=R5=R6=1.0 Ω

- The circuit contains multiple junctions and cannot be reduced using simple series–parallel rules alone.

- Some resistors connect the same pairs of nodes, and symmetry may be used to simplify the analysis.

- Kirchhoff’s Laws are required.

(a) Identify all unique nodes in the circuit and label them clearly.

(b) Determine which resistors (if any) carry equal currents, and explain why.

(c) Determine the current through each resistor.

(d) Determine the voltage drop across each resistor.

(e) Determine the total current supplied by the battery.

(f) Determine the equivalent resistance of the circuit.

- A 100 V DC power source is connected to a circuit consisting of nine resistors, arranged as shown in the diagram. The circuit contains multiple junctions and loops and cannot be simplified using basic series–parallel reductions alone.

Given information

- R1=3 Ω

- R2=3 Ω

- R4=2 Ω

- R5=2 Ω

- R7=1 Ω

- R8=1 Ω

Additional measurements:

- The current through R2is IR2=6.0 A

- The voltage drop across R5is VR5=6.0 V

- The current through R9is IR9=1.0 A

The resistances of R3, R6, and R9are unknown.

(a) Identify and clearly label all junctions (nodes) in the circuit.

(b) Determine:

- the current through R1

- the current through R4

- the current through R7

(c) Determine the voltage drop across R2.

(d) Determine the resistance of R3.

(e) Determine the resistance of R6.

(f) Determine the resistance of R9.

(g) Determine the total current supplied by the battery.

Solutions